Инструкция по эксплуатации AVD2.

Общеее описание.

AVD2 – это аналоговый квадратурный демодулятор, предназначенный для работы в диапазоне 700-2700 МГц.

Общий вид блока AVD2  представлен на рисунке 1.

Структурная схема представлена на рисунке 2.

У AVD2 есть вход информационного сигнала RFIN (XW1). На него подаётся сигнал диапазона 700-2700 МГц. При желании возможно засинхронизировать внутренний генератор AVD2 от внешнего сигнала. Для этого на вход EXT_REF (XW2) необходимо подать сигнал с частотой 100 МГц и мощностью 0...10 дБм.

Квадратурный демодулированный сигнал с полосой до 100 МГц (суммарной до 200 МГц по I и Q каналу) снимаеться с выходов I_OUT (XW3) и Q_OUT (XW4).

X1 – разъём питания и управления.

Номера контактов и функциональное назначение интерфейсов приведены в таблице 1.

Таблица 1.

(разъем штыревой 16 контактов, шаг 2 мм.).

| № контакта | Обозначение | Описание |
| --- | --- | --- |
| 1 | SPI_DATA | Вход сигнала DATA конфигурационного SPI интерфейса (возможен вариант двунаправленной шины) |
| 2 | GND |  |
| 3 | SPI_SS | Вход сигнала SS конфигурационного SPI интерфейса |
| 4 | -12В | Питание -12V, (1) |
| 5 | SPI_CLK | Вход сигнала CLK конфигурационного SPI интерфейса |
| 6 | -12В | Питание -12V, (1) |
| 7 | DATAOUT | Дополнительный сигнала конфигурационного SPI интерфейса (в данное время не используется) |
| 8 | GND |  |
| 9 | +5В | Питание +5V, (2) |
| 10 | GND |  |
| 11 | +5В | Питание +5V, (2) |
| 12 | GND |  |
| 13 | +12В | Питание +12V, (3) |
| 14 | GND |  |
| 15 | +12В | Питание +12V, (3) |
| 16 | GND |  |

Примечание: для уменьшения нагревания блока AVD2 питающие напряжения +12В, -12 В желательно снизить до +9 В и -9 В соответственно.

- общий макс. ток -12В – 0.2А

- общий макс. ток  +5В – 0.7А

- общий макс. ток  +12В – 1.2А

Характеристики.

| Parameter | Conditions/subparameters | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Frequency |  |  |  |  |
| Frequency range |  | 700 | 2700 | MHz |
| Frequency resolution |  |  | 1 | Hz |
| Frequency accuracy | T=0..+70oC |  | ±10 | ppm |
| External reference |  | 100 |  | MHz |
|  | Input Resistance | 50 |  | Ω |
|  | Input level | 0 | 10 | dBm |
| Synthesizer setting time | (any step width) |  | 5 | ms |
| Phase noise | @1GHz, @Offset (Hz): |  |  |  |
|  | 100 |  | -90 | dBc/Hz |
|  | 1k |  | -95 | dBc/Hz |
|  | 10k |  | -95 | dBc/Hz |
|  | 100k |  | -95 | dBc/Hz |
|  | 1M |  | -105 | dBc/Hz |
| RF Input |  |  |  |  |
| Input Resistance |  | 50 |  | Ω |
| Input level | normal operation | -80 | 0 | dBm |
|  | nonlinear | +10 |  | dBm |
| Maximum composite signal level | normal operation |  | +35 | dBc |
|  | nonlinear |  | 0 | dBm |
| Gain Control | total range | 100 |  | dB |
|  | step size |  | 1 | dB |
| VSWR @50Ω | 700..2700 MHz |  | 2 |  |
|  | 950..2150 MHz |  | 1.5 |  |
| LO reradiation |  | -90 |  | dBm |
| Noise Figure |  |  | 7 | dB |
| IIP3 | (input stage) | +17 |  | dBm |
| Input P1dB | (input stage) | +10 |  | dBm |
| Baseband (I/Q) |  |  |  |  |
| I/Q Filter 1 | 3dB cutoff | 104 |  | MHz |
|  | DC-100 MHz flatness |  | 1.5 | dBpp |
|  | rejection band (40dB) | 167 |  | MHz |
|  | group delay flatness (DC-50MHz) |  | 6 | ns (p-p) |
| I/Q Filter 2 | bandpass (3dB) | 15.9 |  | MHz |
|  | rejection band (40dB) | 23 |  | MHz |
|  | group delay flatness (DC-10MHz) |  | 12 | ns (p-p) |
| I/Q Filter 3 | bandpass (3dB) | 1.96 |  | MHz |
|  | rejection band (40dB) | 3.2 |  | MHz |
|  | group delay flatness (DC-2MHz) |  | 0.3 | us (p-p) |
| I/Q Filter 4 | bandpass (3dB) | 450 |  | kHz |
|  | rejection band (40dB) | 770 |  | kHz |
|  | group delay flatness (DC-0.4MHz) |  | 1.2 | us (p-p) |
| SINAD | Pin=-45..-10, @10MHz BW | 40 | 50 | dB |
|  | Pin=-35..-15, @10MHz BW | 40 | 60 | dB |
| Image rejection |  | 40 | 55 | dB |
| I/Q outputs | Voltage swing |  | 1 | Vpp |
|  | DC offset | -50 | +50 | mV |
|  | output impedance |  | 1 | Ω |
|  | nominal load | 500 |  | Ω |
| Power Supply |  |  |  |  |
| +9VA |  | +9 | +12 | V |
| +9VA supply current |  |  | 1.2 | A |
| +5VA |  | +4.75 | +5.25 | V |
| +5VA supply current |  |  | 0.7 | A |
| -12VA |  | -12 | -9 | V |
| -12VA supply current |  |  | 0.2 | A |
| Mechanical Data |  |  |  |  |
| Dimensions | Width | 87.5±0.2 |  | mm |
|  | Height | 12.5+0.5 |  | mm |
|  | Depth (w/o connectors) | 185±0.3 |  | mm |
| Connectors | In/Out | Connector Type |  | Side |
|  | RF Input | SMA, female |  | front |
|  | Ext. Osc. Reference | SMA, female |  | front |
|  | I-channel output | SMA, female |  | back |
|  | Q-channel output | SMA, female |  | back |
|  | SPI control | Pin holder, 2x8=16pin, 2mm pitch |  | back |
| Total Power Dissipation |  |  | 15 | W |